UGOT Python common errors

This is a non-exhaustive list of problems that students often have when working with the UGOT.

Common Python errors, such as syntax or indentation errors, are mostly not included. However, you should also be aware of them, especially if you learned Python a long time ago and have forgotten how easy it is to use the wrong type of brackets, miss a colon, or use lowercase when it should be uppercase.

- InactiveRpcError – a problem with connection between laptop and UGOT. Check that the device and UGOT are connected to the same Wifi network (or that the device is connected to the UGOT hotspot), and that the IP address string is correct. The IP address will change if the Wifi network / method of connection changes.

- ModuleNotFoundError – unless there is a typo in the imports, one or more of the packages is missing, in which case you should run `pip install packagename1 packagename2`, either in a new Jupyter notebook cell or in the terminal (with the correct environment activated, if using). The complete list is: `pip install ugot opencv-python numpy ultralytics mediapipe`.

- NoneTypeError – in the context of opening cv2 windows for the UGOT camera view, this often occurs when either the UGOT camera was not opened (check for got.open_camera()) or the camera failed to send data (sometimes just a single frame). In the latter case, you can add an `if frame:` condition so that numpy / cv2 will not process the frame unless there is some data. In a minority of cases, it could also be an issue with the camera wire – this is usually the case if the same error occurs multiple times.

- Python does not show any errors, but the robot is not moving – on the UGOT, go to Settings -> Switch model and select the correct model. Check also that the commands in the code are suitable for the model, e.g. the engineer vehicle should have movement commands like got.mecanum_move_speed() and not got.balance_move_speed().

- The self-balancing car tries to move but cannot balance itself – sometimes this just happens, and you will have to prop it up or hold it before running the code, but sometimes it’s because there is no got.balance_start_balancing() and/or a bit of a delay with time.sleep() before other movement commands.