# UGOT Commands

## Setup

ugot.UGOT()
Creates an instance of the robot controller UGOT class. The rest of this document assumes your instance is stored in a variable called got.

got.initialize(device_ip: str) -> None
Connects to the robot at the given IP address (e.g. ‘192.168.1.189’).

## AprilTag recognition

got.load_models(models: list) -> bool
Loads one or more perception models (here, ['apriltag_qrcode']) so the robot can detect AprilTags.

got.get_apriltag_total_info() -> list
Returns a list of lists, where each inner list represents one AprilTag. Each inner list contains: the ID, centre x-coordinate, centre y-coordinate, height, width, area, distance (assuming 5x5), distance (assuming 7x7), distance (assuming 10x10), pose angle x, pose angle y, pose angle z, horizontal bearing angle, vertical bearing angle.

## Display screen

got.screen_clear() -> None
Clears any text or graphics currently shown on the robot’s onboard display.

got.screen_display_background(color: int) -> None
Fills the display background with a solid colour (e.g. 3 = red, 6 = green) as a status indicator.

## Mechanical arm movement

got.mechanical_joint_control(angle1: int, angle2: int, angle3: int, duration: int) -> None
Moves the arm joints to angles (degrees) in the given duration (milliseconds). Joint 1 is the closest joint to the main body while joint 3 is the furthest.

got.mechanical_clamp_close() -> None
Closes the gripper (clamp) to grasp an object.

got.mechanical_clamp_release() -> None
Opens the gripper.

## Chassis movement

got.mecanum_move_speed_times(direction: int, speed: int, times: int, units: int) -> None
Drives the mecanum wheel car forward / backward for a set time.

- direction: 0 is forwards, 1 is backwards

- speed: [5-80] in cm/s

- times: [0-360] seconds or centimetres

- units: 0 is seconds, 1 is centimetres

got.mecanum_turn_speed_times(turn: int, speed: int, times: int, unit: int) -> None
Rotates the robot in place by a given angle at a given speed, repeated the specified number of times.

- turn: 2 is left, 3 is right

- speed: [5-280] in degrees/s

- times: [0-360] seconds or degrees

- unit: 0 is seconds, 2 is degrees

got.mecanum_move_xyz(x_speed: int, y_speed: int, z_speed: int) -> None
Sends a one-shot velocity command in the robot’s X (left / right), Y (forward / backward), and Z (rotation) axes.

got.mecanum_stop() -> None
Immediately halts all mecanum wheel motion.